# Wireless Sample 2 User’s Manual

Wireless Sample 2  (WS2) is located in $TOSROOT/apps/WS2-ARQ.

## Installation

The WS2 Imote2 firmware must be installed on each mote that is attached to the structure being monitored.

- cd $TOSROOT/apps/WS2-ARQ

- make imote2 debug

  - ignore the make error messages at the end

- USBLoaderHost.exe –p build/imote2/main.bin.out

  - Make sure the USB cable is connected directly to the mote.

TOSBase must be installed on the mote that is attached to the computer.

- cd $TOSROOT/apps/TOSBase

- make imote2 debug

  - ignore the make error messages at the end

- USBLoaderHost.exe –p build/imote2/main.bin.out

  - Make sure the USB cable is connected directly to the mote (not to the debug board).

The Java WS2 application serves as the user interface for controlling the wireless network.

- cd $TOSROOT/apps/WS2-ARQ/java

- make

## Executon

After installing the WS2 firmware on the Imote2s, and TOSBase on the gateway Imote2, perform the following to launch the application:

- cd $TOSROOT/apps/WS2-ARQ/java

- java net.tinyos.sf.SerialForwarder –comm serial@COM3:115200 &

- java WS2

Note that you must change COM3 to whatever serial port your gateway mote is attached to as specified in Device Manager.

You should see the following user interface:

- Ensure that the start and end search frequencies as shown on the top of the user interface are correct.

- Ensure that the sampling frequency of each mote is correct.

- Note that each of the

- Hit the “Set Init Values” of each mote.  Observe the green LED blinking on the mote.

- Hit the “Start DLAC !!!” button.

  - You will see the blue LED blink 6 times, pause, and then blink 6 times again.  It will repeat this each time it takes a set of accelerometer readings.

  - Once it is done performing the root calculation, it will turn on the green LED.

  - You must wait for the green LED to turn on before continuing!

  - The red LED will blink if an error condition has occurred.

- Once the green LED has turned on, you can hit the “Get Result” button on each mote, which will retrieve the root calculations.

- To validate, you can also click the “Get Raw Data” button on each mote, which will grab the raw sensor readings.

- The Get PS Data will return a portion of the power spectrum data for validation purposes.

- Click on “Save Results” to save the data to a file.  Note the command line output to see where the files are saved.

## Control Knobs

### Sampling Frequencies

The sampling frequency of each mote is specified in $TOSROOT/apps/WS2-ARQ/java/MoteSettings.txt.  This file contains two tab-separated columns: the first is the Mote ID in hexadecimal notation (it is the last two digits of the mote’s label), the second column is the sampling frequency in Hz.

### Start and End Search Frequencies

The start and end search frequencies are specified in $TOSROOT/apps/WS2-ARQ/java/StatEndFreq.txt.  This contains 5 tab-separated columns and two rows.  The first row specifies the start frequencies and the second row specifies the end frequencies.

### Number of Samples to Average

- Open $TOSROOT/apps/WS2-ARQ/ControllerP.nc and edit NUM_AVG on line 24.

- You must re-compile and re-upload WS2-ARQ onto the motes for this to take effect.

### Wait Interval Between Samples

- Open $TOSROOT/apps/WS2-ARQ/ControllerP.nc and edit AVG_WAIT on line 25.

- The value is in binary milliseconds (1024 = 1s).

- The default value is 1024.

- You must re-compile and re-upload WS2-ARQ onto the motes for this to take effect.

## LED Behavior

- WS2 rapidly blinks the blue LED 6 times every time it starts taking a set of raw accelerometer readings.

- WS2 turns on the green LED when it is done taking the samples, averaging the power spectrum, and calculating the roots.

- WS2 blinks the red LED whenever an error has occurred